HACETTEPE UNIVERSITY

ELECTRICAL AND ELECTRONICS ENGINEERING

ELE432 - Advanced Digital Design

HOMEWORK 2-Multicycle Processor Controller

Name & Surname: BERKE MALÇOK

Student ID: 2210357081

,

Q1)  
I spent approximately 12 hours completing this lab. The most time-consuming part was analyzing the synchronization between the testbench's clock edges and the FSM state transitions to ensure the outputs matched the expected test vectors perfectly.

Q2)
The full hierarchical design consists of the controller (top module), maindec (FSM), aludec (ALU Control), and instrdec (Immediate Decoder).

code

CONTROLLER

module controller( input logic clk, input logic reset, input logic [6:0] op, input logic [2:0] funct3, input logic funct7b5, input logic zero, output logic [1:0] immsrc, output logic [1:0] alusrca, output logic [1:0] alusrcb, output logic [1:0] resultsrc, output logic adrsrc, output logic [2:0] alucontrol, output logic irwrite, output logic pcwrite, output logic regwrite, output logic memwrite ); logic [1:0] aluop; logic branch; logic pcupdate; maindec md( .clk(clk), .reset(reset), .op(op), .alusrca(alusrca), .alusrcb(alusrcb), .resultsrc(resultsrc), .adrsrc(adrsrc), .irwrite(irwrite), .regwrite(regwrite), .memwrite(memwrite), .branch(branch), .pcupdate(pcupdate), .aluop(aluop) ); aludec ad( .opb5(op[5]), .funct3(funct3), .funct7b5(funct7b5), .aluop(aluop), .alucontrol(alucontrol) ); instrdec id( .op(op), .immsrc(immsrc) ); assign pcwrite = pcupdate ¦ (branch & zero); endmodule

MAINDEC

module maindec( input logic clk, input logic reset, input logic [6:0] op, output logic [1:0] alusrca, output logic [1:0] alusrcb, output logic [1:0] resultsrc, output logic adrsrc, output logic irwrite, output logic regwrite, output logic memwrite, output logic branch, output logic pcupdate, output logic [1:0] aluop ); typedef enum logic [3:0] { S0_FETCH = 4'd0, S1_DECODE = 4'd1, S2_MEMADR = 4'd2, S3_MEMREAD = 4'd3, S4_MEMWB = 4'd4, S5_MEMWRITE = 4'd5, S6_EXECUTER = 4'd6, S7_ALUWB = 4'd7, S8_EXECUTEI = 4'd8, S9_JAL = 4'd9, S10_BEQ = 4'd10 } statetype; statetype state, nextstate; always_ff @(posedge clk or posedge reset) begin if (reset) state <= S0_FETCH; else state <= nextstate; end always_comb begin case (state) S0_FETCH: nextstate = S1_DECODE; S1_DECODE: begin case (op) 7'b0000011: nextstate = S2_MEMADR; // lw 7'b0100011: nextstate = S2_MEMADR; // sw 7'b0110011: nextstate = S6_EXECUTER; // R-type 7'b0010011: nextstate = S8_EXECUTEI; // I-type ALU 7'b1101111: nextstate = S9_JAL; // jal 7'b1100011: nextstate = S10_BEQ; // beq default: nextstate = S0_FETCH; endcase end S2_MEMADR: begin case (op) 7'b0000011: nextstate = S3_MEMREAD; // lw 7'b0100011: nextstate = S5_MEMWRITE; // sw default: nextstate = S0_FETCH; endcase end S3_MEMREAD: nextstate = S4_MEMWB; S4_MEMWB: nextstate = S0_FETCH; S5_MEMWRITE: nextstate = S0_FETCH; S6_EXECUTER: nextstate = S7_ALUWB; S7_ALUWB: nextstate = S0_FETCH; S8_EXECUTEI: nextstate = S7_ALUWB; S9_JAL: nextstate = S7_ALUWB; S10_BEQ: nextstate = S0_FETCH; default: nextstate = S0_FETCH; endcase end always_comb begin alusrca = 2'b00; alusrcb = 2'b00; resultsrc = 2'b00; adrsrc = 1'b0; irwrite = 1'b0; regwrite = 1'b0; memwrite = 1'b0; branch = 1'b0; pcupdate = 1'b0; aluop = 2'b00; case (state) S0_FETCH: begin alusrca = 2'b00; alusrcb = 2'b10; resultsrc = 2'b10; adrsrc = 1'b0; irwrite = 1'b1; pcupdate = 1'b1; regwrite = 1'b0; memwrite = 1'b0; branch = 1'b0; aluop = 2'b00; end S1_DECODE: begin alusrca = 2'b01; alusrcb = 2'b01; resultsrc = 2'b00; adrsrc = 1'b0; irwrite = 1'b0; pcupdate = 1'b0; regwrite = 1'b0; memwrite = 1'b0; branch = 1'b0; aluop = 2'b00; end S2_MEMADR: begin alusrca = 2'b10; alusrcb = 2'b01; resultsrc = 2'b00; adrsrc = 1'b0; irwrite = 1'b0; pcupdate = 1'b0; regwrite = 1'b0; memwrite = 1'b0; branch = 1'b0; aluop = 2'b00; end S3_MEMREAD: begin alusrca = 2'b00; alusrcb = 2'b00; resultsrc = 2'b00; adrsrc = 1'b1; irwrite = 1'b0; pcupdate = 1'b0; regwrite = 1'b0; memwrite = 1'b0; branch = 1'b0; aluop = 2'b00; end S4_MEMWB: begin alusrca = 2'b00; alusrcb = 2'b00; resultsrc = 2'b01; adrsrc = 1'b0; irwrite = 1'b0; pcupdate = 1'b0; regwrite = 1'b1; memwrite = 1'b0; branch = 1'b0; aluop = 2'b00; end S5_MEMWRITE: begin alusrca = 2'b00; alusrcb = 2'b00; resultsrc = 2'b00; adrsrc = 1'b1; irwrite = 1'b0; pcupdate = 1'b0; regwrite = 1'b0; memwrite = 1'b1; branch = 1'b0; aluop = 2'b00; end S6_EXECUTER: begin alusrca = 2'b10; alusrcb = 2'b00; resultsrc = 2'b00; adrsrc = 1'b0; irwrite = 1'b0; pcupdate = 1'b0; regwrite = 1'b0; memwrite = 1'b0; branch = 1'b0; aluop = 2'b10; end S7_ALUWB: begin alusrca = 2'b00; alusrcb = 2'b00; resultsrc = 2'b00; adrsrc = 1'b0; irwrite = 1'b0; pcupdate = 1'b0; regwrite = 1'b1; memwrite = 1'b0; branch = 1'b0; aluop = 2'b00; end S8_EXECUTEI: begin alusrca = 2'b10; alusrcb = 2'b01; resultsrc = 2'b00; adrsrc = 1'b0; irwrite = 1'b0; pcupdate = 1'b0; regwrite = 1'b0; memwrite = 1'b0; branch = 1'b0; aluop = 2'b10; end S9_JAL: begin alusrca = 2'b01; alusrcb = 2'b10; resultsrc = 2'b00; adrsrc = 1'b0; irwrite = 1'b0; pcupdate = 1'b1; regwrite = 1'b0; memwrite = 1'b0; branch = 1'b0; aluop = 2'b00; end S10_BEQ: begin alusrca = 2'b10; alusrcb = 2'b00; resultsrc = 2'b00; adrsrc = 1'b0; irwrite = 1'b0; pcupdate = 1'b0; regwrite = 1'b0; memwrite = 1'b0; branch = 1'b1; aluop = 2'b01; end default: begin alusrca = 2'b00; alusrcb = 2'b00; resultsrc = 2'b00; adrsrc = 1'b0; irwrite = 1'b0; regwrite = 1'b0; memwrite = 1'b0; branch = 1'b0; pcupdate = 1'b0; aluop = 2'b00; end endcase end endmodule

ALUDEC

module aludec( input logic opb5, input logic [2:0] funct3, input logic funct7b5, input logic [1:0] aluop, output logic [2:0] alucontrol ); logic rtypesub; assign rtypesub = funct7b5 & opb5; always_comb begin case (aluop) 2'b00: alucontrol = 3'b010; // add 2'b01: alucontrol = 3'b110; // sub default: begin case (funct3) 3'b000: alucontrol = rtypesub ? 3'b110 : 3'b010; // sub / add-addi 3'b010: alucontrol = 3'b111; // slt 3'b110: alucontrol = 3'b001; // or 3'b111: alucontrol = 3'b000; // and default: alucontrol = 3'b010; endcase end endcase end endmodule

INSTRDEC

module instrdec( input logic [6:0] op, output logic [1:0] immsrc ); always_comb begin case (op) 7'b0110011: immsrc = 2'bxx; // R-type 7'b0010011: immsrc = 2'b00; // I-type ALU 7'b0000011: immsrc = 2'b00; // lw 7'b0100011: immsrc = 2'b01; // sw 7'b1100011: immsrc = 2'b10; // beq 7'b1101111: immsrc = 2'b11; // jal default: immsrc = 2'b00; endcase end endmodule

Q3)
Yes, the implemented controller successfully passes all test vectors provided in the controller.tv file without any errors.

The testbench samples the outputs at the negative clock edge (0.065 ns). As seen in the waveform, the controller's actual output perfectly matches the expected value (16'hXa2c). This confirms that the FSM state transitions and decoder logics are timed and functioning exactly as required.